ROOFING SPECIFICATION PREPARED BY SIPLAST, INC.

PARAGUARD PERIMETER METAL SYSTEMS

This specification is provided as a general guide for use of Siplast products based on typical building conditions and standard roofing practices.  Siplast is strictly a manufacturer of roofing systems and has no experience, training or expertise in the areas of architecture/engineering or in the area of consulting with respect to matters related to such areas.  Siplast recommends that the Owner's representative independently verify the accuracy and appropriateness of a specification provided for a specific project.

December 1, 2022

SECTION 07 71 00 ROOF SPECIALTIES

PART 1  GENERAL

  - SECTION INCLUDES:

- Preparation of surfaces to receive factory fabricated metal perimeter and vegetated accessory systems.

- Installation of factory fabricated and finished metal perimeter and vegetated accessory systems.

  - RELATED SECTIONS

- Section [----] - Rough Carpentry

- Section 03 52 00 - Lightweight Concrete Roof Insulation

- Section [----] - Rigid Roof Insulation

- Section 07 52 00 - Modified Bituminous Membrane Roofing

- Section 07 54 19 – Polyvinyl Chloride Roofing

- Section [----] - Modified Bitumen Membrane Waterproofing

- Section [----] - Vegetated Overburden System

  - REFERENCE STANDARDS

NRCA	National Roofing Contractors Association

Rosemont, IL

OSHA	Occupational Safety and Health Administration

Washington, DC

SMACNA	Sheet Metal and Air Conditioning Contractors National Association

Chantilly, VA

FM	Factory Mutual Engineering and Research

Norwood, MA

ANSI	American National Standards Institute

Washington, DC

SPRI	Single Ply Roofing Industry

Waltham, MA

  - SUBMITTALS

- Submittals Prior to Contract Award:

  - Submit a letter from the roofing/waterproofing membrane manufacturer confirming that the factory fabricated metal accessory systems furnished for the project are supplied or manufactured by the roofing/waterproofing membrane manufacturer and that each component section is labeled with the roofing membrane manufacturer’s logo.

  - Latest edition of factory fabricated metal component manufacturer/supplier’s installation instructions for the factory fabricated metal perimeter system(s).

  - Samples from the manufacturer/supplier sized to represent metal components.

  - Copies of the manufacturer/supplier's color selection chart showing the manufacturer/supplier's full range of standard colors as well as physical samples of each standard color.

  - Sample copy of the roofing system manufacturer's inclusion addendum offering coverage of the factory fabricated metal perimeter system(s) as part of the roofing/waterproofing membrane system.

  - QUALITY ASSURANCE

- Agency Approvals:  The proposed factory fabricated metal component shall conform to the following requirements.  No other testing agency approvals will be accepted.

- The roof perimeter fascia and coping system(s) shall be certified through third party verification by the manufacturer/supplier to meet performance design criteria according to the most recent edition of ANSI/SPRI/FM 4435/ES-1: Wind Design Standard for Edge Systems Used with Low Slope Roofing Systems.

*NOTE: Verify the Windstorm Classification for the appropriate metal component dimension and gauge per the third party testing agency’s ANSI/SPRI/FM 4435/ES-1 test report for the specified factory fabricated metal roof perimeter systems.

- Scope of Work:  The work to be performed under this specification shall include but is not limited to the following:  Attend necessary job meetings and furnish competent and full time supervision, experienced mechanics, all materials, tools, and equipment necessary to complete, in an acceptable manner, the factory fabricated metal installation in accordance with this specification.  Comply with the latest written application instructions of the manufacturer/supplier of the factory fabricated metal components.

- Local Regulations:  Conform to regulations of public agencies, including any specific requirements of the city and/or state of jurisdiction.

- Manufacturer Requirements:

- Ensure that the factory fabricated metal components are labeled with the roofing membrane manufacturer’s logo.

- Ensure that the factory fabricated metal component manufacturer/supplier provides direct trained company personnel to attend necessary job meetings, perform periodic inspections as necessary, and conducts a final inspection upon successful completion of the project.

  - PRODUCT DELIVERY STORAGE AND HANDLING

- Delivery:  Deliver materials in the manufacturer's original packaging.

- Storage:  Store materials out of direct exposure to the elements.

- Strippable Film Masking:  Do not remove the strippable film masking on the metal component until immediately following installation.  Do not allow extended UV or heat exposure to metal components covered with strippable film masking.

- Damaged Material:  Any materials that are found to be damaged will be automatically rejected, removed and replaced at the Contractor's expense.

  - PROJECT/SITE CONDITIONS

- Requirements Prior to Job Start

- Related Work:  Verify that all related work performed by other trades is complete prior to installing the factory fabricated metal components.

- Component Substrate Condition:  Mounting surfaces shall be straight and secure and provide adequate widths to properly support the factory fabricated metal components.

- Safety:  Familiarize every member of the application crew with all safety regulations recommended by OSHA, SMACNA and other industry or local governmental groups.

- Protection Requirements

- Component Protection:  Protect newly applied factory fabricated metal component surfaces against mechanical damage.

- Limited Access:  Prevent access by the public to materials, tools and equipment during the course of the project.

- Debris Removal:  Remove all debris daily from the project site.

- Site Condition:  Complete, to the owner's satisfaction, all job site clean-up including building interior, exterior and landscaping where affected by construction.

  - GUARANTEE/ ADDENDUM

- Roof Membrane Guarantee Addendum:  In addition to the specified guarantee under section 07 52 00, furnish the Owner with the roofing/waterproofing manufacturer's inclusion addendum to the guarantee offering coverage of the factory fabricated raised edge, gravel stop, extruded fascia, coping, expansion joint, separator, perforated gravel stop, drain cover, perimeter blocking systems under the standard terms of the roofing/waterproofing membrane/system guarantee.

>	Siplast Paraguard Roof Perimeter System Inclusion Addendum

*   NOTE: Preselect metal component type(s).

PART 2  PRODUCTS

  - DESCRIPTION OF FACTORY FABRICATED METAL SYSTEMS

- Factory Fabricated Metal Coping System:  Metal Coping components shall be factory fabricated according to the requirements of the roofing membrane manufacturer and labeled with the roofing manufacturer’s logo.  The metal coping system shall consist of the following components:

-	Factory formed anchor/cleat plates fabricated from 16 gauge, G90 galvanized steel.

-	Factory formed splice plates with embossed ribs to prevent lateral water flow at joints between adjacent sections fabricated from 0.032 inch aluminum and having factory applied sealant strips.

- A factory formed coping cap fabricated from minimum 24 gauge galvanized steel 0.050 inch aluminum 0.063 inch aluminum 24 gauge stainless steel 22 gauge stainless steel having a mill coil coated Kynar™ spray coated Kynar™ clear anodized, bronze anodized, black anodized finish.

-	Factory formed miters and end caps.

>	Paraguard L Coping System, by Siplast, Inc., Dallas, TX (800) 922-8800

- Factory Fabricated Metal Coping System:  Metal Coping components shall be factory fabricated according to the requirements of the roofing membrane manufacturer and labeled with the roofing manufacturer’s logo.  The metal coping system shall consist of the following components:

-	Factory formed anchor/cleat plates fabricated from 20 gauge 16 gauge, G90 galvanized steel.

-	Factory formed splice plates fabricated from 0.032 inch aluminum with factory applied sealant strips.

- A factory formed coping cap fabricated from minimum 24 gauge galvanized steel 0.050 inch aluminum 0.063 inch aluminum 24 gauge stainless steel 22 gauge stainless steel having a mill coil coated Kynar™ spray coated Kynar™ clear anodized, bronze anodized, black anodized finish.

-	Factory formed miters and end caps.

>	Paraguard M Coping System, by Siplast, Inc., Dallas, TX (800) 922-8800

- Factory Fabricated Metal Coping System:  Metal Coping components shall be factory fabricated according to the requirements of the roofing membrane manufacturer and labeled with the roofing manufacturer’s logo.  The metal coping system shall consist of the following components:

-	Factory formed continuous inside and outside cleats fabricated from G90 galvanized steel.

-	Factory formed splice plates fabricated from 0.032 inch aluminum with factory applied sealant strips.

- A factory formed coping cap fabricated from minimum 24 gauge galvanized steel 0.050 inch aluminum 0.063 inch aluminum 24 gauge stainless steel 22 gauge stainless steel having a mill coil coated Kynar™ spray coated Kynar™ clear anodized, bronze anodized, black anodized finish.

-	Factory formed miters and end caps.

>	Paraguard M Continuous Cleat Coping System, by Siplast, Inc., Dallas, TX (800) 922-8800

- Factory Fabricated Metal Coping System:  Metal Coping components shall be factory fabricated according to the requirements of the roofing membrane manufacturer and labeled with the roofing manufacturer’s logo.  The metal coping system shall consist of the following components:

-	Factory formed anchor cleats fabricated from G90 galvanized steel consisting of a 16 gauge base with a 20 gauge riser.  Cleat must be capable of spanning an unsupported exterior wall space of up to 6 inches

-	Factory formed splice plates fabricated from 0.032 inch aluminum with factory applied sealant strips.

- A factory formed coping cap fabricated from minimum 24 gauge galvanized steel 0.050 inch aluminum 0.063 inch aluminum 24 gauge stainless steel 22 gauge stainless steel having a mill coil coated Kynar™ spray coated Kynar™ clear anodized, bronze anodized, black anodized finish.

-	Factory formed miters and end caps.

>	Paraguard Cantilever Coping System, by Siplast, Inc., Dallas, TX (800) 922-8800

- Factory Fabricated Fascia:  Fascia components shall be factory formed according to the requirements of the membrane manufacturer and labeled with the roofing manufacturer’s logo.  The fascia system shall consist of the following components:

-	An extruded aluminum anchor bar with pre-punched slotted fastening holes, secured using stainless steel hex head fasteners provided by the manufacturer.

-	Pre-formed EPDM anchor bar spacers.

- A factory formed exterior fascia, fabricated from minimum 24 gauge galvanized steel 0.050 inch aluminum 0.063 inch aluminum 24 gauge stainless steel 22 gauge stainless steel having a mill coil coated Kynar™ spray coated Kynar™ clear anodized, bronze anodized, black anodized finish.

-	Factory formed folded miters and end caps.

>	Paraguard Extruded Edge AT System, by Siplast, Inc., Dallas, TX (800) 922-8800

- Factory Fabricated Fascia:  Fascia components shall be factory formed according to the requirements of the membrane manufacturer and labeled with the roofing manufacturer’s logo.  The fascia system shall consist of the following components:

-	An extruded aluminum anchor bar with pre-punched slotted fastening holes, secured using stainless steel hex head fasteners provided by the manufacturer.

-	Factory formed splice plates.

- A factory formed exterior fascia, fabricated from minimum 24 gauge galvanized steel 0.050 inch aluminum 0.063 inch aluminum 24 gauge stainless steel 22 gauge stainless steel having a mill coil coated Kynar™ spray coated Kynar™ clear anodized, bronze anodized, black anodized finish.

-	Factory formed folded miters and end caps.

>	Paraguard Parasolo Drip Edge System, by Siplast, Inc., Dallas, TX (800) 922-8800

- Factory Fabricated Raised Roof Edge:  Factory fabricated raised roof edge components shall be factory formed according to the requirements of the membrane manufacturer and labeled with the roofing manufacturer’s logo.  The raised roof edge system shall consist of the following components:

-	A factory formed cant dam fabricated from 24 gauge, G90 galvanized steel, secured using galvanized ringshank roofing nails.

-	A factory formed retaining rail with pre-punched fastener slots, fabricated from 22 gauge, G90 galvanized steel, secured using using #14 fasteners as supplied by the manufacturer.

-	A factory formed stainless steel retention spring clips designed for long term support of the exterior fascia engagement.

- A factory formed exterior fascia, fabricated from minimum 24 gauge galvanized steel 0.050 inch aluminum 0.063 inch aluminum 24 gauge stainless steel 22 gauge stainless steel having a mill coil coated Kynar™ spray coated Kynar™ clear anodized, bronze anodized, black anodized finish.

-	Factory formed concealed splice plates.

-	Factory formed miters and end caps.

-	Factory formed leveling supports for accurate cant dam installation.

>	Paraguard M Raised Edge System, by Siplast, Inc., Dallas, TX (800) 922-8800

- Factory Fabricated Gravel Stop:  Factory fabricated gravel stop components shall be factory formed according to the requirements of the membrane manufacturer and labeled with the roofing manufacturer’s logo.  The gravel stop system shall consist of the following components:

-	A factory formed cleat with pre-punched nail holes fabricated from 22 gauge, G90 galvanized steel, secured using galvanized ringshank roofing nails.

- A factory formed gravel stop with pre-punched nailing holes, secured using galvanized roofing nails. fabricated from minimum 24 gauge galvanized steel 0.050 inch aluminum 0.063 inch aluminum 24 gauge stainless steel having a mill coil coated Kynar™ spray coated Kynar™,clear anodized, bronze anodized, black anodized finish.

-	Factory formed concealed splice plates.

-	Factory formed welded miters.

>	Proform Gravel Stop, by Siplast, Inc., Dallas, TX (800) 922-8800

- Factory Fabricated Fascia Extender:  Factory fabricated fascia extender components shall be factory formed according to the requirements of the membrane manufacturer and labeled with the roofing manufacturer’s logo.  The fascia extender system shall consist of the following components:

-	A factory formed cleat with pre-punched nail holes fabricated from 24 gauge, G90 galvanized steel, secured using galvanized ringshank roofing nails.

- A factory formed exterior fascia extender with pre-punched nailing holes, secured using galvanized roofing nails. Fabricated from minimum 24 gauge galvanized steel 0.050 inch aluminum 0.063 inch aluminum 24 gauge stainless steel 22 gauge stainless steel having a mill coil coated Kynar™ spray coated Kynar™ clear anodized, bronze anodized, black anodized finish.

-	Factory formed concealed splice plates.

-	Factory formed welded miters.

>	Proform Fascia Extender, by Siplast, Inc., Dallas, TX (800) 922-8800

- Factory Fabricated Roof-To-Roof-Expansion Joint:  Factory fabricated roof-to-roof expansion joint components shall be factory formed according to the requirements of the membrane manufacturer and labeled with the roofing manufacturer’s logo.  The roof-to-roof expansion joint system shall consist of the following components:

- A factory formed cap fabricated from minimum 24 gauge galvanized steel 0.050 inch aluminum 0.063 inch aluminum 24 gauge stainless steel 22 gauge stainless steel having a mill coil coated Kynar™ spray coated Kynar™ clear anodized, bronze anodized, black anodized finish.

-	A factory formed travel cleat fabricated from 20 gauge, G90 galvanized steel.

Specially designed shouldered washers to allow for travel cleat movement.

-	Factory formed splice plates fabricated from 0.040 inch aluminum with factory applied sealant strips.

-	Factory formed curb rails fabricated from 24 gauge, G90 galvanized steel.

-	Factory formed welded miters, end caps, tees, and crosses.

>	Paraguard Roof-To-Roof Expansion Joint System, by Siplast, Inc., Dallas, TX (800) 922-8800

- Factory Fabricated Roof-To-Wall Expansion Joint:  Factory fabricated roof-to-wall expansion joint components shall be factory formed according to the requirements of the membrane manufacturer and labeled with the roofing manufacturer’s logo.  The roof-to-wall expansion joint system shall consist of the following components:

- A factory formed cap fabricated from minimum 24 gauge galvanized steel 0.050 inch aluminum 0.063 inch aluminum 24 gauge stainless steel 22 gauge stainless steel having a mill coil coated Kynar™, spray coated Kynar™ clear anodized, bronze anodized, black anodized finish.

-	A factory formed travel cleat fabricated from 20 gauge, G90 galvanized steel.

-	Specially designed shouldered washers to allow for travel cleat movement.

-	Factory formed splice plates fabricated from 0.040 inch aluminum with factory applied sealant strips.

-	Factory formed curb rail fabricated from 24 gauge, G90 galvanized steel.

-	Factory formed welded miters and end caps.

>	Paraguard Roof-To-Wall Expansion Joint System, by Siplast, Inc., Dallas, TX (800) 922-8800

- Factory Fabricated Perimeter Blocking:  Factory fabricated metal perimeter blocking components shall be factory formed according to the requirements of the membrane/lightweight insulating concrete manufacturer and labeled with the roofing/lightweight insulating concrete manufacturer’s logo.  The metal perimeter blocking system shall consist of the following components:

- A factory formed slotted base, side panel, and cap components fabricated from minimum 20 gauge, G90 galvanized steel designed to infill the perimeter blocking system with lightweight insulating concrete.

-	Factory formed braces fabricated from 20 gauge, G90 galvanized steel.

-	Adjustable tapered support angle to allow adjustment to the desired slope.

-	Factory formed shelf clips to facilitate proper seating of braces and the cap component.

>	Paraguard Eliminailer C Perimeter Blocking System, by Siplast, Inc., Dallas, TX (800) 922-8800

PART 3 EXECUTION

  - SUBSTRATE PREPARATION

- Perimeter Nailers:  Perimeter nailers shall be flat and level to the building perimeter edge.  The front edge of the nailer must be flush with the outside face or wall of the building.  Anchor all perimeter nailers in strict accordance with the guidelines set forth in FM Global Property Loss Prevention Data Sheet 1-49.

- Curbs for Expansion Joint Components:  Curbs must be straight, level, and properly anchored to the building structural deck.  Any curbs, which are improperly installed or anchored, must be corrected prior to installation of the expansion joint systems.

- Flashing Membrane Installation:  Ensure that all roofing flashing treatments used in conjunction with factory fabricated metal components are installed according to the roofing membrane manufacturer's specifications, current technical guide, and details prior to installation of the factory fabricated metal component.

- Surface Cleaning:  Sweep or vacuum all surfaces to receive the metal components, removing all loose aggregate, soil, and foreign substances prior to installation of the factory fabricated metal components.

  - FACTORY FABRICATED METAL COMPONENT INSTALLATION

- Install metal components in accordance with the roofing/waterproofing manufacturer's instructions and the following requirements.

- Factory Fabricated Metal Coping Installation.

* NOTE	The following text is applicable to Paraguard L Coping, Paraguard M Coping, and Paraguard Cantilever Coping only.

- Place corner support clips at all corners to support the cap.  Set the coping system manufacturer's support clips at the corner and fasten in accordance with the coping system manufacturer's installation instructions.

- Beginning at the corners and/or ends, install the anchor cleats, splice plates, and factory formed miters and end cap components in strict accordance with the coping system manufacturer’s installation instructions.

- Position all remaining anchor cleats for straight sections of coping in strict accordance with the factory fabricated coping system manufacturer's installation instructions and code approval requirements, pulling each cleat snugly against the exterior face of the building.

- Install splices centered on the anchor cleats in accordance with the coping system manufacturer's installation instructions.

- Install full length sections of coping cap by hooking the outside leg of the coping cap over the outside face of the cleats first.  Rotate the cap over the top of the wall pressing lightly, but firmly, on the top of the cap until the inside leg fully locks over the roof side of the anchor cleats.  Allow a 3/8 inch gap between coping sections for thermal movement.

- Factory Fabricated Metal Coping Installation.

* NOTE	The following text is applicable to Paraguard M Continuous Cleat Coping, only.

- Set outside and inside continuous cleats beginning at corners and/or ends.  Position all cleats using a splice plate as a spacer in strict accordance with the factory coping system manufacturer's installation instructions and code approval requirements.

- Install splices centered into the end of coping sections, miters, end caps, and end wall terminations in accordance with the coping system manufacturer's installation instructions.

- Beginning again at the corners and/or ends, hook the outside leg of the coping cap over the outside continuous cleat first.  Rotate the cap over the top of the wall pressing lightly, but firmly, on the top of the cap until the inside leg fully locks over the inside continuous cleat.  Field crimp the drip on the inside leg of the coping cap at splice joints and at 18 inches on center.  Allow a 3/8 inch gap between coping sections for thermal movement.

- Factory Fabricated Fascia

* NOTE	The following text is applicable to Paraguard Extruded Edge AT only.

1.	After completion of the installation of the roofing membrane plies, apply a three inch wide, 1/4 inch thick band of the roofing manufacturer’s specified mastic or sealant to the underside of the flange of the extruded aluminum anchor bar.  Install the anchor bar joint spacer into the end of the extruded aluminum anchor bar to allow the next section to fit properly over the anchor bar joint spacer.  Secure the extruded aluminum anchor bar to the perimeter nailer in accordance with the roof system manufacturer's installation instructions.

2.	Beginning again at the corners, hook the top of the exterior fascia onto the top of the extruded aluminum anchor bar and apply slight pressure downward until the fascia engages over the bottom of the extruded aluminum anchor bar.  Position the next section to overlap the preceding section approximately 1 inch at the notches provided.  Install sections of the fascia cover from right to left as viewed from the rooftop engaging the fascia cover over the anchor bar in the same method as previously described.

- Factory Fabricated Fascia

* NOTE	The following text is applicable to Paraguard Parasolo Drip Edge only.

1.	After completion of the installation of the roofing membrane plies, apply a three inch wide, 1/4 inch thick band of the roofing manufacturer’s specified mastic or sealant to the underside of the flange of the extruded aluminum anchor bar.  Install the anchor bar joint spacer into the end of the extruded aluminum anchor bar to allow the next section to fit properly over the anchor bar joint spacer.  Secure the extruded aluminum anchor bar to the perimeter nailer in accordance with the roof system manufacturer's installation instructions.

2.	Beginning again at the corners, hook the bottom of the exterior fascia onto the bottom of the extruded aluminum anchor bar and rotate into place applying slight pressure until the fascia is fully engaged.  Position the next section to overlap the preceding section approximately 1 inch at the notches provided.  Install sections of the fascia cover from right to left as viewed from the rooftop engaging the fascia cover onto the anchor bar in the same method as previously described.

- Factory Fabricated Raised Roof Edge

1.	Install the factory fabricated water dam over the base ply of roof membrane with levelers positioned 2 inches from the water dam ends and one leveler in the center of 12 foot length sections.  Secure the water dam to the perimeter nailer in accordance with the raised roof edge system manufacturer's installation instructions.

2.	After completion of the installation of the roofing and flashing membrane plies over the water dam, beginning at miters, place the retaining rail over the finished flashing membrane firmly, without forcing.  The retaining rail shall be level and the nailing slots shall align centered with the nailer underneath the membrane/water dam assembly.  Fasten the retaining rail in accordance with the raised roof edge system manufacturer's installation instructions.

3	Place factory formed spring clips over the retaining rail at 4 feet on center, ensuring that there will be a spring clip under each fascia cover end.

4.	Beginning again at the corners, place splice plates into each end of the exterior fascia cover and install the exterior fascia by setting it onto the retaining rail and firmly pushing down until the fascia engages to the bottom of the retaining rail.  Allow a 3/8 inch gap between raised roof edge sections for thermal movement.

- Factory Fabricated Gravel Stop

1.	Place the continuous retainer cleat to the roofing surface firmly against the perimeter nailer.  The retainer cleat should be level and the nailing slots should align centered with the nailer underneath.  Fasten the retaining cleat in accordance with the gravel stop system manufacturer's installation instructions.

2.	Starting at the corners, trowel a bead of the roofing manufacturer's specified mastic over the base ply of membrane where the flange of the exterior fascia is to be set.  Hook the drip edge of the exterior fascia over the retainer cleat and fasten the flange through the pre-punched holes in accordance with the gravel stop system manufacturer's installation instructions.  Slide a concealed joint splice plate halfway into the fascia to allow the next section to fit halfway over the joint splice plate as well.  Allow a 3/8 inch gap between gravel stop sections for thermal movement.

3.	After installation of the factory fabricated gravel stop is complete, ensure that the roofing stripping and finish plies are installed in accordance with the roofing membrane manufacturer's specifications and details.

- Factory Fabricated Fascia Extender

1.	Anchor the continuous galvanized clip to the wall surface fastened at 12 inches on center.

2.	Hook the drip edge of the fascia extender over the continuous clip.  Use joint splice plates behind adjoining sections of fascia extender.  Fasten the top flange of the fascia extender at 12 inches on center.  Allow a 3/8 inch gap between fascia extender sections for thermal movement.

- Factory Fabricated Roof to Roof Expansion Joint

1.	Fasten the curb rails to the previously flashed curbs using roofing nails every 24 inches on center.

2.	Place the traveler cleat over the curb rails, allowing the horizontal slots to center over the rail.  Secure the travel cleat in accordance with the expansion joint system manufacturer's installation instructions.

3.	Splice plates are placed over the travel cleat on 12 foot centers, ensuring that a splice plate is placed centered where there will be a joint in the cap sections.

4.	Hook the bottom edge of the cap face of one side securely on the drip edge of the traveler cleat.  While maintaining engagement, rotate the cap length over the cleat and press the cap firmly downward on the back edge above the traveler cleat until it locks onto the cleat.  Allow a 3/8 inch gap between expansion joint sections for thermal movement.

- Factory Fabricated Roof to Wall Expansion Joint

1.	Fasten the curb rail to the previously flashed curb using roofing nails spaced 24 inches on center.

2.	Fasten the wall rail to the wall using fasteners approved for use with the wall substrate every 24 inches on center.

3.	Place the travel cleat over the curb rail, allowing the horizontal slots to center over the rail.  Secure the travel cleat through the slots using the drill point screws and extruded washers to the curb first and then to the wall centered over the installed rails.

4.	Place guttered splice plates over the traveler cleat on 12 foot centers, ensuring that a splice plate is placed centered where there will be a joint in the cap sections.

5.	Hook the bottom edge of the cap face securely on the drip edge of the traveler cleat.  While maintaining engagement, rotate the cap length over the cleat positioning the top leg of the cap firmly against the wall.  Fasten the top leg of the cap to the wall using fasteners approved for use with the wall substrate at 12 inches on center.  Flash or seal the top leg of the expansion joint cap in accordance with the roofing membrane manufacturer's specifications and details.  Allow a 3/8 inch gap between expansion joint sections for thermal movement.

- Factory Fabricated Perimeter Blocking

1.	Place the steel braces into the base and slide into the pre-punched shelf clips.  Fasten each brace into position from the back of the base component through the steel braces in accordance with the manufacturer’s installation instructions.

2.	Move the base component into position and fasten it through the pre-punched holes into the structural deck.

3.	Install the adjustable support angles to the desired height and angle by fastening them to the steel braces per the manufacturer’s installation instructions.

4.	Continue installation of adjacent sections of base components repeating the same process previously described prior to installation of the remaining components of the perimeter blocking system.  Tightly abut adjacent sections of the base component to one another.

5.	Place the side panel into position and fasten it to the steel braces at the rate specified by the manufacturer.  Tightly abut adjacent sections of side panel to one another.

6.	Insert the retainer clip onto the giant shelf clip on the backside of the base component per the manufacturer’s installation instructions and place the cap component into position.  Ensure that the flange on the rear of the top engages the giant shelf clips and fasten the cap component into the steel braces, adjustable support angles, and side panel as required by the manufacturer.  Tightly abut adjacent sections of the cap component to one another.

  - FIELD QUALITY CONTROL AND INSPECTIONS

- Site Condition:  Leave all areas around the job site free of debris, construction materials, equipment and related items after completion of job.

- Issuance Of The Addendum to the Roofing Membrane/System Guarantee:  Complete all post installation procedures and meet the factory fabricated metal manufacturer/supplier's final endorsement for issuance of the addendum to the specified roofing/waterproofing guarantee.

END OF SECTION 07 71 00